Source of Production:  Nissan Motor Co., Ltd.

Assembly Plant Location: Tochigi, Japan

| ENGINE |  |
| --- | --- |
| Type | VQ37VHR DOHC 24-valve |
| Orientation | Vertical |
| Cylinders / configuration | V6 with coated pistons |
| Block / head composition | Aluminum / aluminum |
| Displacement (liters / cc) | 3.7 liters / 3696cc |
| Horsepower | 330 hp @ 7,000 rpm |
| Torque | 270 lb-ft. @5,200 rpm |
| Bore x stroke (mm) | 95.5 x 86.0 |
| Compression ratio (:1) | 11.0 |
| Maximum engine speed (indicated redline on tachometer) | 7,500 rpm |
| Induction system | Sequential multi-point fuel injection |
| Valvetrain | 4 valves per cylinder |
| Mounts (sides/rear) | 2 solid/ 1 liquid-filled |
| Recommended fuel | Premium unleaded |
| Emissions system | Closed loop feedback system with 3-way catalyst |
| Emission certification level | 50 State, LEV2 |
| Exhaust | Dual tail tubes with dual chrome finishers |

| ELECTRICAL |  |
| --- | --- |
| Electronic ignition system | S |
| 130 amp alternator |  |
| 1.6 kW starter |  |
| 80D23L battery |  |

| DRIVETRAIN | ### G37 Journey | ### Sport 6MT | ### G37x AWD |
| --- | --- | --- | --- |
| Drive configuration | Front engine/rear-wheel drive |  | Front engine/ all-wheel drive |
| Vehicle Dynamic Control (VDC) | S |  |  |
| Viscous Limited-Slip Differential (VLSD) | O | S | O |
| Transmission type | 7-speed auto w/DS mode and Downshift Rev Matching | 6-speed manual | 7-speed auto w/DS mode and Downshift Rev Matching |
| Electronic Automatic Transmission Control | S | - | S |

| DRIVETRAIN (Cont.) |  | ### G37 Journey | ### Sport 6MT | ### G37x AWD |
| --- | --- | --- | --- | --- |
| Gear ratios (:1) |  | 7 A/T | 6 M/T | 7 A/T |
|  | 1st | 4.923 | 3.794 | 4.923 |
|  | 2nd | 3.193 | 2.324 | 3.193 |
|  | 3rd | 2.042 | 1.624 | 2.042 |
|  | 4th | 1.411 | 1.271 | 1.411 |
|  | 5th | 1.000 | 1.00 | 1.000 |
|  | 6th | 0.862 | 0.794 | 0.862 |
|  | 7th | 0.771 | - | 0.771 |
|  | Reverse | 3.972 | 3.446 | 3.972 |
| Final drive ratio (:1) |  | 3.357 | 3.692 | 3.357 |

| BODY, SUSPENSION AND STEERING |  | ### G37 Journey | ### Sport 6MT | ### G37x AWD |
| --- | --- | --- | --- | --- |
| Body type |  | Corrosion-resistant high-strength steel unibody with aluminum hood |  |  |
| Front suspension |  | Double wishbone w/ stabilizer bar |  |  |
| Rear suspension |  | Independent multi-link w/ stabilizer bar |  |  |
| Sport-tuned suspension |  | P | S | P |
| Steering type |  | Rack-and-pinion with vehicle-speed-sensitive power steering system |  |  |
| Steering ratio (:1) |  | 16.4 standard, 14.7 Sport (RWD only) |  |  |
| Turning diameter (curb-to-curb) feet |  | 36.1 (RWD), 36.7 (AWD) |  |  |
| Turns (lock-to-lock) |  | 3.1 standard, 2.7 Sport, 2.9 AWD |  |  |

| BRAKES |  | ### G37 Journey | ### Sport 6MT | ### G37x AWD |
| --- | --- | --- | --- | --- |
| Front Size (inches) (diameter x thickness) | Type 13.0 x 1.3 | Power-assisted vented disc |  |  |
|  |  | S | - | S |
|  | 14.0 x 1.3 | P | S | P |
| Rear Size (inches) (diameter x thickness) | Type 13.0 x 0.6 | Power-assisted vented disc |  |  |
|  |  | S | - | S |
|  | 13.8 x 0.8 | P | S | P |
| Electronic Brake force Distribution (EBD) |  | S |  |  |
| Brake Assist (BA) |  | S |  |  |
| Anti-lock Braking System (ABS) |  | 4-wheel/4-channel/4-sensor |  |  |

| WHEELS AND TIRES | ### G37 Journey | ### Sport 6MT | ### G37x AWD |
| --- | --- | --- | --- |
| 18x8-inch aluminum-alloy | S | - | S |
| Split V-spoke 19x8.5-inch front, 19x9.0-inch rear aluminum-alloy | P | S | - |
| Split V-spoke 19x8.5 aluminum-alloy | - |  | P |
| P225/50R18 94V, all season tires | S | - | S |
| Fr: P225/45R19 92W; Rr: P245/40R19 94W, summer tires | P | S | - |
| P225/45R19 all-season tires | - | - | P |
| Temporary spare tire in trunk | S |  |  |

| EXTERIOR DIMENSIONS (in inches) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Wheelbase |  |  | 112.2 |  |  |
| Overall length |  |  | 183.1 | 183.7 | 183.1 |
| Overall width |  |  | 71.8 |  |  |
| Overall height | 18-inch tires 19-inch tires |  | 54.8 54.9 | - 54.9 | 55.4 55.4 |
| Front track width |  |  | 60.8 |  |  |
| Rear track width |  |  | 61.4 |  |  |
| Coefficient of Drag (Cd) |  |  | 0.31 | 0.32 | 0.31 |

| EXTERIOR FEATURES AND OPTIONS | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| High Intensity Discharge (HID) projector-beam xenon headlights | S |  |  |
| Auto on/off headlights | S |  |  |
| Body-color dual power remote-controlled outside mirrors | S |  |  |
| Fog lights | S |  |  |
| LED taillights | S |  |  |
| Heated outside mirrors | S |  |  |
| Body-color grille | P | S | P |
| Sport front fascia, side sills | P | S | P |
| Rear decklid spoiler | - | P | P |
| Power slide & tilt glass moonroof with one-touch open/close and sliding sunshade | P | S | P |

| INTERIOR DIMENSIONS (inches) | Front | Rear |
| --- | --- | --- |
| Head room | 39.4 – w/o moonroof 37.7 – w/ moonroof | 34.5 |
| Shoulder room | 53.7 | 52.7 |
| Hip room | 53.4 | 48.2 |
| Leg room | 43.8 | 29.8 |

| INTERIOR FEATURES AND OPTIONS | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| Leather-appointed seats | S |  |  |
| Heated front seats | S |  |  |
| 8-way power driver’s seat with manual adjustable lumbar support | S |  |  |
| Front sport seats with manual thigh extension and driver’s power torso bolster and thigh support adjustment | P | S | P |
| Driver’s power lumbar support | P | S | P |
| 8-way power passenger’s seat | S |  |  |
| “Silk Obi” finish aluminum trim | S |  |  |
| Solid magnesium paddle shifter | P | - | P |
| Folding rear seat back | S |  |  |
| Front seatback map pockets | S |  |  |
| Rear window defroster with indicator light and timer | S |  |  |
| Rear head restraints | S |  |  |
| Illuminated entry system w/time delay fade-out | S |  |  |
| Memory system for drivers seat, steering wheel and outside mirrors | P | S | P |

| AUDIO AND NAVIGATION SYSTEMS | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| 6-speaker audio system with RDS, in-dash CD autochanger with MP3/WMA playback capability | S | - | S |
| Infiniti Studio on Wheels® by Bose® Premium Audio system w/ RDS, 11 speakers, in-dash single CD with Driver’s Audio Stage | P | S | P |
| 7.0-inch information display color monitor | S |  |  |
| USB connectivity for iPod®, flash drive or other compatible storage device | S |  |  |
| Rear window diversity antenna | S |  |  |
| SiriusXM Satellite Radio (SiriusXM subscription required, sold separately) | S |  |  |
| Infiniti Hard Drive Navigation System with NavTraffic and NavWeather (SiriusXM subscription required, sold separately), Zagat Survey® | P | S | P |

| # INSTRUMENTATION |  |
| --- | --- |
| Speedometer, tachometer, coolant temperature and fuel level analog gauges | S |
| Center display: HVAC status, outside temperature and compass |  |
| Digital dual trip odometers |  |
| VDC off switch |  |
| Analog clock |  |

| COMFORT AND CONVENIENCE FEATURES | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| Cruise control w/ steering wheel mounted controls | S |  |  |
| Intelligent Cruise Control (ICC) w/wheel-mounted controls | P | - | P |
| Bluetooth® Hands-free Phone System | S |  |  |
| Power tilt and telescopic steering wheel | P | S | P |
| Aluminum pedals | P | S | P |
| Rear Sonar System | - | S | - |
| RearView Monitor | S |  |  |
| Automatic temperature control |  |  |  |
| Dual-zone automatic temperature control |  |  |  |
| Auto-up/down and auto-reverse feature front power windows (keyfob operable) |  |  |  |
| Power door locks (driver and passenger side switches) with auto door lock system |  |  |  |
| Tilt meters – meters move up and down with steering wheel tilt |  |  |  |
| Locking glove compartment with lights |  |  |  |
| Dual overhead map lights with sunglasses holder |  |  |  |
| Driver and front passenger sunvisors w/ extenders and illuminated vanity mirrors |  |  |  |
| 4 front/2 rear cupholders |  |  |  |
| Front seat belt guides |  |  |  |
| Infiniti Intelligent Key System |  |  |  |

| # CAPACITIES | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| Seating capacity | 4 |  |  |
| Interior passenger volume (cu. ft.) – SAE Method | 92.4 (w/o moonroof); 90.1 (w/ moonroof) |  |  |
| Trunk volume (cu. ft.) | 7.4 |  |  |
| Fuel tank (gals.) | 20.0 |  |  |
| Engine oil (qts.) | 4.9 |  |  |
| Engine coolant (qts.) | 9.0 | 9.1 | 9.0 |

| SAFETY AND SECURITY FEATURES |  |
| --- | --- |
| Advanced Air Bag System (AABS) with dual-stage supplemental front air bags with seat belt and occupant classification sensors | S |
| Roof-mounted curtain side impact supplemental air bags for front and rear-seat outboard occupant head protection |  |
| Front seat-mounted side impact supplemental air bags |  |
| LATCH System (Lower Anchors and Tethers for CHildren) |  |
| Vehicle Dynamic Control (VDC) and Traction Control System (TCS) |  |
| Zone Body Construction with front and rear crumple zones |  |
| Energy-absorbing steering column |  |
| Hood buckling creases and safety stops |  |
| Pipe-style steel side door guard beams |  |
| Slide-away brake pedal assembly |  |
| Reinforced side structure |  |
| Power window lockout button |  |
| Emergency inside trunk release |  |
| Driver and front passenger knee bolsters |  |
| 3-point front seat belts with pretensioners and load limiters |  |
| Front-seat Active Head Restraints |  |
| Vehicle Security System (VSS) |  |
| Tamper resistant door lock key cylinder |  |
| Remote Keyless Entry system |  |
| Tire Pressure Monitoring System (TPMS) |  |

| CURB WEIGHTS |  | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- | --- |
| Curb weight (lbs.) | Front Rear Total | 1,971 1,662 3,633 | 1,993 1,715 3,708 | 2,143 1,691 3,834 |
| Weight distribution (% fr/rr) |  | 54/46 | 54/46 | 56/44 |
| GVWR (lbs.) |  | 4,656 | 4,656 | 4,830 |

| EPA MILEAGE ESTIMATES (Actual mileage may vary) | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| City | 19 | 17 | 18 |
| Highway | 27 | 25 | 25 |
| EPA Classification | Subcompact |  |  |

| # PACKAGES AND OPTIONS | G37 Journey | Sport 6MT | G37x AWD |
| --- | --- | --- | --- |
| Premium Package: Power sliding glass moonroof with one-touch open/close feature; Infiniti Studio on Wheels® by Bose® Premium Audio system with CD player with MP3 playback capability, Radio Data System (RDS) and 11 speakers; Drivers Audio Stage; memory system for driver’s seat, steering wheel and outside mirrors; driver’s seat, steering wheel and outside mirror position synchronization; power tilt and telescopic steering wheel; driver’s seat power lumbar support (2-way), Rear Sonar System | P | S | P |
| Navigation Package: Infiniti Hard Drive Navigation System with enhanced graphics, touchscreen, Lane Guidance and 3-D building graphics, Zagat Survey® restaurant reviews, NavTraffic and NavWeather (SiriusXM subscription required, sold separately), Bluetooth® Advanced Audio Distribution Profile; DVD-video playback, Infiniti Voice Recognition for climate control, audio and navigation systems | P | S | P |
| Technology Package: Intelligent Cruise Control, Brake Assist with Preview Braking, Pre-Crash Seat Belts, rain-sensing windshield wipers, Advanced Climate Control System (ACCS) with Plasmacluster air purifier, Grape polyphenol filter | P | - | P |
| Sport Package: 19x8.5-inch split V-spoke aluminum-alloy wheels with PP225/45R19 summer performance tires (front); 19x9.0-inch aluminum-alloy wheels with P245/40R19 summer performance tires (rear) RWD; 19x8.5-inch aluminum-alloy wheels with P225/45R19 all-season tires - AWD; sport-tuned suspension; 14.7:1 sport-tuned steering gear (16.4:1 AWD); Viscous Limited-Slip Differential (VLSD); unique sport front fascia; front sport seats with manual thigh extension and driver’s power torso and thigh support adjustment; unique sport stitching on steering wheel and front seats; aluminum pedals and foot rest; solid magnesium paddle shifters | P | - | P |

| Performance Tire & Wheel Package: Split V-spoke 19x8.5-inch aluminum-alloy wheels with PP225/45R19 summer performance tires (front); 19x9.0-inch aluminum-alloy wheels with P245/40R19 W-rated summer performance tires (rear) | P | S | - |
| --- | --- | --- | --- |
| Interior Accents Package: High-gloss Maple accents on center console, instrument panel and doors | O | - | O |

O – Option

S – Standard

P – Package

7/12