Source of Production:  Nissan Motor Co., Ltd.

Assembly Plant Location: Tochigi, Japan

| ENGINE |  |
| --- | --- |
| Type | VQ37VHR DOHC 24-valve |
| Orientation | Vertical |
| Cylinders / configuration | V6 |
| Block / head composition | Aluminum / aluminum |
| Displacement (liters / cc) | 3.7 liters / 3696cc |
| Horsepower | 343 @ 7,400 rpm |
| Torque | 273 @5,200 rpm |
| Bore x stroke (mm) | 95.5 x 86.0 |
| Compression ratio (:1) | 11.0 |
| Maximum engine speed | 7,500 rpm |
| Induction system | Sequential multi-point fuel injection |
| Valvetrain | 4 valves per cylinder |
| Mounts (sides/rear) | 2 liquid-filled / 1 liquid-filled |
| Required fuel | Premium unleaded |
| Emissions system | Closed loop feedback system with 3-way catalysts |
| Emission certification level | 50 State, LEV2 |
| Exhaust | IPL-tuned high-flow dual exhaust, H-pipe design with dual round chrome finishers |
| IPL badged engine cover | S |

| ELECTRICAL |  |
| --- | --- |
| Electronic ignition system | S |
| 150 amp alternator |  |
| 1.6 kW starter |  |
| 80D23L battery |  |

| DRIVETRAIN |  |  |
| --- | --- | --- |
| Drive configuration |  | Front engine/rear-wheel drive |
| Vehicle Dynamic Control (VDC) |  | S |
| Transmission type |  | 7-speed automatic w/DS mode and Downshift Rev Matching, solid magnesium paddle shifters |
| Gear ratios (:1) |  | 7A/T |
|  | 1st | 4.923 |
|  | 2nd | 3.193 |
|  | 3rd | 2.042 |
|  | 4th | 1.411 |
|  | 5th | 1.000 |
|  | 6th | 0.862 |
|  | 7th | 0.771 |
|  | Reverse | 3.972 |
| Final drive ratio (:1) |  | 3.357 |

| BODY, SUSPENSION AND STEERING |  |  |
| --- | --- | --- |
| Body type |  | Corrosion-resistant high-strength steel unibody with aluminum hood |
| Front suspension |  | Independent double wishbone w/ stabilizer bar |
| Rear suspension |  | Independent multi-link w/ stabilizer bar |
| IPL-tuned performance suspension |  | S |
| Steering type |  | Rack-and-pinion with vehicle-speed-sensitive power steering system |
| Sport-tuned steering |  | S |
| Steering ratio (:1) |  | 14.7 |
| Turning diameter (curb-to-curb) feet |  | 36.1 |
| Turns (lock-to-lock) |  | 2.73 |

| BRAKES |  |  |
| --- | --- | --- |
| Front Rotor size (inches) | Type 14.0x1.3 | Power-assisted vented disc, 4-psiton opposed calipers |
|  |  | S |
| Rear Rotor size (inches) | Type 13.8 x 0.8 | Power-assisted vented disc, 2-piston opposed calipers |
|  |  | S |
| Electronic Brake force Distribution (EBD) |  | S |
| Brake Assist |  | S |
| Anti-lock Braking System (ABS) |  | 4-channel/4-sensor |
| Parking brake |  | Foot type |

| ###### WHEELS AND TIRES |  |
| --- | --- |
| #### Wheels |  |
| IPL split 7-spoke graphite finish 19x8.5-inch front, 19x9.0-inch rear aluminum-alloy | S |
| #### Tires |  |
| Fr: P225/45R19 96W; Rr: P245/40R19 98W, Bridgestone Potenza RE050A summer tires | S |
| Spare tire repair kit | S |
| Tire Pressure Monitoring System | S |

| EXTERIOR DIMENSIONS (inches) |  |  |  |
| --- | --- | --- | --- |
| Wheelbase |  |  | 112.2 |
| Overall length |  |  | 185.3 |
| Overall width |  |  | 72.9 |
| Overall height |  |  | 55.1 |
| Front track width |  |  | 60.8 |
| Rear track width |  |  | 62.6 |
| Coefficient of Drag (Cd) |  |  | 0.31 |

| EXTERIOR FEATURES AND OPTIONS |  |
| --- | --- |
| IPL aerodynamics (front and rear fascias, side sills, rear spoiler) | S |
| High Intensity Discharge (HID) bi-functional xenon headlights | S |
| Auto on/off headlights | S |
| Body-color dual power folding, heated remote-controlled outside mirrors with 2-position memory | S |
| 3-piece automatically retracting power hardtop | S |
| IPL exterior badging | S |

| INTERIOR DIMENSIONS (inches) | Front | Rear |
| --- | --- | --- |
| Head room (top up) | 39.4 | 34.5 |
| Shoulder room | 53.7 | 45.8 |
| Hip room | 53.4 | 41.7 |
| Leg room | 43.9 | 27.2 |

| INTERIOR FEATURES AND OPTIONS |  |  |
| --- | --- | --- |
| Leather-appointed seats with IPL logo (front) | S |  |
| Climate controlled front seats w/cooling function (deletes seatback map pocket) | S |  |
| 8-way power driver’s seat with manual adjustable lumbar support | S |  |

| Driver’s seat power lumbar support (2-way) | S |
| --- | --- |
| 8-way power passenger’s seat | S |
| Maple accents |  |
| Aluminum pedals and foot rest | S |
| Magnesium paddle shifters | - |
| Front seatback map pockets |  |
| Rear window defroster with indicator light and timer | S |
| Illuminated entry system with time delay fade-out | S |
| Driver’s seat and steering wheel memory (2 position) | S |
| Driver’s seat, steering wheel and outside mirror position synchronization | S |
| Power tilt and telescopic steering wheel and steering column | S |

| # CAPACITIES |  |
| --- | --- |
| Seating capacity | 4 |
| Interior passenger volume (cu. ft.) | 78.4 |
| Trunk volume (cu. ft., for vehicles without Spare Tire Kit, SAE measurements) | 10.33 top up/1.99 top down |
| Fuel tank (gals.) | 20 |
| Engine oil (qts.) | 4.9 |
| Engine coolant (qts.) | 9.0 |

| CURB WEIGHTS |  |  |
| --- | --- | --- |
| Curb weight (lbs.) | Front Rear Total | 2,151 2,038 4,189 |
| Weight distribution (% fr/rr) |  | 51/49 |
| GVWR (lb) |  | 5,079 |

| AUDIO AND NAVIGATION SYSTEMS |  |  |  |
| --- | --- | --- | --- |
| Bose® Open Air Sound System with noise adaptive technology, AM/FM, RDS and 13 speakers, including Bose® front seat Personal Speakers | S |  |  |
| Infiniti Hard Drive Navigation System with NavTraffic and NavWeather (SiriusXM subscription required, sold separately), Zagat Survey®, Streaming Audio via Bluetooth® | S |  |  |
| DVD-video playback | S |  |  |
| USB input for iPod®, flash drive or other compatible storage devices | S |  |  |
| SiriusXM Satellite Radio (SiriusXM subscription required, sold separately) | S |  |  |
| 7.0-inch information display color monitor | S |  |  |
| Infiniti Voice Recognition for navigation, audio and climate control functions | S |  |  |

| COMFORT AND CONVENIENCE |  |
| --- | --- |
| Cruise control w/ steering wheel-mounted controls | S |
| Bluetooth® Hands-free Phone System | S |
| RearView Monitor |  |
| Adaptive Dual Zone Climate Control Sys. |  |
| Auto-up/down front power windows (key fob operable) with auto reverse feature |  |
| One-touch auto down rear windows |  |
| Power door locks (driver and passenger side switches) with auto lock system |  |
| Locking glove compartment with lights |  |
| Dual overhead map lights |  |
| Driver and front passenger sunvisors and illuminated vanity mirrors |  |
| 4 front cupholders |  |
| Infiniti Intelligent Key with Push Button Ignition |  |
| Analog clock with night illumination |  |
| Power walk-in device with position memory for easy access to 2nd row seats |  |

| EPA MILEAGE ESTIMATES (Actual mileage may vary) |  |
| --- | --- |
| City | 17 |
| Highway | 25 |
| EPA Classification | Subcompact |

| SAFETY AND SECURITY FEATURES |  |
| --- | --- |
| Advanced Air Bag System (AABS) with dual-stage front supplemental air bags with seat belt and occupant classification sensors | S |
| Door-mounted curtain side impact supplemental air bags |  |
| Front seat-mounted side impact supplemental air bags |  |
| Dual Rear Pop-up Roll bars |  |
| 4-Wheel Anti-lock Braking System (ABS) |  |
| Vehicle Dynamic Control (VDC) and Traction Control System (TCS) |  |
| Zone Body Construction with front and rear crumple zones |  |
| Energy-absorbing steering column |  |
| Hood buckling creases and safety stops |  |
| Pipe-style steel side door guard beams |  |
| Slide-away brake pedal assembly |  |
| Power window lockout button |  |
| Emergency inside trunk release |  |
| Driver and front passenger knee bolsters |  |
| 3-point front seat belts (driver’s side ELR, passenger side ALR/ELR), pretensioners and load limiters |  |
| Front-seat Active Head Restraints |  |
| Vehicle Security System (VSS) with indicator light |  |
| Infiniti Vehicle Immobilizer System |  |
| Tamper resistant door lock key cylinder |  |
| Remote Keyless Entry system |  |

7/12